Communication systems Lab 5

Harshal Abhyankar 2019A3PS0282P

Task 1

from scipy import stats

import scipy

from numpy import sqrt

from scipy.integrate import quad

import numpy as np

# Uniform distribution

a,b = 0,2

def uniform(x):

return 1 / (b - a)

uniform_verify = quad(uniform, a, b)

# Gaussian distribution

mu,sigma = 0,1

def gauss(x):

return scipy.stats.norm.pdf(x, mu, sigma)

gaussian_verify = quad(gauss, -np.inf, np.inf)

# Rayleigh Distribution

alpha = 2

def ray(x):

return scipy.stats.rayleigh.pdf(x, alpha)

ray_verify = quad(ray, 0, np.inf)

# Exponential Distribution

beta = 2

def expdist(x):

return scipy.stats.expon.pdf(x, beta)

expdist_verify = quad(expdis, 0, np.inf)

print("Uniform Distribution verification:", uniform_verify)

print("Gaussian Distribution verification:", gaussian_verify)

print("Rayleigh Distribution verification:", ray_verify)

print("Exponential Distribution verification:", expdist_verify)

Task 2

Code:

from scipy import special

import numpy as np

import matplotlib.pyplot as plt

import numpy as np

import matplotlib.pyplot as plt

sigma = 12

mu = 0

x = sigma*np.random.randn(10000) + mu

plt.hist(x)

plt.show()

sigma = 12

mu = 0

iter_max=100000

count=0

for iter in range(iter_max):

nt = sigma*np.random.randn(1) + mu

if nt>sigma:

count+=1

Prob= count/iter_max

average_prob=print(Prob)

qfunc_prob=0.5 - 0.5*special.erf(1/np.sqrt(2))

theoretical_prob=print(qfunc_prob)

import numpy as np

import matplotlib.pyplot as plt

sigma = 12

mu = 0

nt = sigma*np.random.randn(1000) + mu

ac=np.correlate(nt, nt, mode = 'full')

#print(ac)

plt.figure(2)

plt. plot(ac)

plt.show()

Histogram:

Comment: As we increase the number of iterations, the actual value will get closer to the theoretical value in the above screenshot.

ACF correlation:

Task 3

Code:

import random

import time

from  numpy.fft import fft

import matplotlib.pyplot as plt

import numpy as np

for T in range(30):

start_time_sinc=-10

stop_time_sinc=10

N=10

fm=N

fs=10*fm

ts=1/fs

time=np.arange(start_time_sinc,stop_time_sinc, ts)

m_t = 2*N*np.sinc(2*N*time)

a=1

sigma=0.01 # change this

mu = 0

n_t=sigma*np.random.randn(len(m_t)) + mu

y_t=a*m_t + n_t

m_t=y_t

mf=fft(m_t)/fs

N=len(mf)

mf_abs_sorted=np.fft.fftshift(abs(mf))

freq_axis= np.linspace(-fs/2,fs/2, N)

plt.figure(1)

plt.plot(time+T,m_t)

plt.title("Time Domain")

plt.show()

For sigma=0.01:

For sigma=0.1:

Conclusion: As sigma increases the distortion in the received signal increases which is due to the noise in the channel.